NATIONAL UNIVERSITY OF MANAGEMENT

Faculty of Information Technology

Raspberry Pi Pico W

Getting Started with MQTT

MicroPython

Publish, Subscribe & Connect Embedded Devices to the IoT Cloud

| Subject | Embedded Systems / IoT Communications |
| --- | --- |
| Level | Beginner to Intermediate |
| Duration | Approx. 90 Minutes |
| Language | MicroPython |
| Hardware | Raspberry Pi Pico W + BME280 Sensor / DHT22 Sensor |
| Prerequisite | Wi-Fi setup with MicroPython (Previous Lecture) |
| Primary Source | Random Nerd Tutorials — randomnerdtutorials.com |
| Date | April 2026 |

Contents

1. Lecture Overview & Learning Objectives	5

Learning Objectives	5

2. What Is MQTT? — Protocol Foundations	6

2.1  Why MQTT for IoT?	6

2.2  MQTT vs Other IoT Protocols	7

3. MQTT Core Concepts	7

3.1  Publish / Subscribe	7

3.2  Messages	8

3.3  Topics	8

3.4  The Broker	10

3.5  Quality of Service (QoS)	11

4. Setting Up the MQTT Broker	11

4.1  Option A — HiveMQ Cloud (Recommended)	12

HiveMQ Setup — Step by Step	12

4.2  Option B — Mosquitto on Raspberry Pi (Local)	14

5. Installing MQTT MicroPython Modules	15

5.1  Installation via Thonny IDE	15

Step 1 — Create the umqtt Directory	15

Step 2 — Create simple.py	16

Step 3 — Create robust.py	17

5.2  Alternative: mpremote mip install	17

5.3  Key umqtt.simple Methods	18

6. Project Setup — config.py & Wi-Fi Helper	19

6.1  config.py — Credentials File	19

6.2  Wi-Fi Connection Helper	20

6.3  Generating a Unique Client ID	20

7. Project A1 — Publishing MQTT Messages (BME280 Sensor)	21

7.1  Hardware Wiring — BME280	21

7.2  Complete Publisher Script	21

Importing Libraries	23

MQTT Topics	24

MQTT Details	24

BME280 Sensor	24

Initialize Wi-Fi	25

Connect to MQTT	25

Publish MQTT Messages	26

Publishing Sensor Readings	26

Testing the Code	27

7.3  Key Code Points	30

Payload Must Be Bytes	30

retain=True Explained	30

SSL for HiveMQ	30

7.4  Testing with MQTT Tools	30

7. Project A2 — Publishing MQTT Messages (DHT22 Sensor)	31

Changes Required	31

Hardware Connection	31

Revised MQTT Publisher Code (DHT22)	32

MQTT Topics Published	34

8. Project B — Subscribing to MQTT Topics (LED Control)	36

8.1  Hardware	36

8.2  Complete Subscriber Script	36

Subscribe to MQTT Topics	38

Callback Function	39

Connecting to Wi-Fi, MQTT Broker and Checking Messages	39

Testing the Code	40

8.3  Critical Subscription Rules	43

Rule 1 — Set Callback Before Subscribe	43

Rule 2 — Compare Bytes, Not Strings	43

check_msg() vs wait_msg()	43

8.4  Sending Test Commands	43

9. Advanced MQTT Features	44

9.1  umqtt.robust — Automatic Reconnection	44

9.2  Retained Messages	44

9.3  Last Will and Testament (LWT)	44

9.4  Multiple Topic Subscriptions & Wildcards	45

9.5  JSON Payloads	45

10. MQTT in Real IoT Systems	46

10.1  Integration Platforms	46

10.2  Node-RED Flow — Receive Pico W Sensor Data	46

10.3  Home Assistant YAML Configuration	46

10.4  Complete System Architecture	47

11. Featured Resource & Further Reading	48

11.1  Continuation Tutorials	48

11.2  Official References	48

12. Summary, Review Questions & Lab Exercises	49

12.1  Key Concepts Summary	49

12.2  Review Questions	49

A — MQTT Concepts	49

B — MicroPython Implementation	49

C — Advanced & Systems	50

12.3  Lab Exercises	50

Lab 1 — Publisher with Random Values (Beginner, 30 min)	50

Lab 2 — LED Subscriber (Beginner, 30 min)	50

Lab 3 — Bidirectional: Publish + Subscribe (Intermediate, 60 min)	50

Lab 4 — Node-RED Dashboard (Advanced, 90 min)	51

12.4  Quick Reference	51

simple.py	52

robust.py	57

# 1. Lecture Overview & Learning Objectives

This lecture introduces MQTT (Message Queuing Telemetry Transport) — the industry-standard lightweight messaging protocol for IoT. Students will master the publish/subscribe model, set up cloud and local MQTT brokers, install MicroPython umqtt modules, and build two real projects: a BME280 sensor data publisher and an LED control subscriber.

### Learning Objectives

- Explain the MQTT publish/subscribe model and its IoT advantages over HTTP.

- Identify the four core concepts: Publish/Subscribe, Messages, Topics, and Broker.

- Set up a cloud MQTT broker (HiveMQ) and a local broker (Mosquitto).

- Install and configure umqtt.simple and umqtt.robust MicroPython modules.

- Create a modular config.py file for Wi-Fi and broker credentials.

- Publish BME280 sensor readings to MQTT topics every 10 seconds.

- Subscribe to MQTT topics and control an LED based on incoming messages.

- Understand QoS levels 0, 1, and 2 and their appropriate use cases.

- Apply MQTT in real IoT architectures with Node-RED and Home Assistant.

📌 Prerequisites - Completion of: Raspberry Pi Pico W — Getting Started with Wi-Fi (MicroPython). - MicroPython firmware flashed. Thonny IDE or VS Code with MicroPython extension installed. - Hardware for full lab: BME280 sensor, LED, 220 Ohm resistor, breadboard, jumper wires. - An active Wi-Fi network (2.4 GHz). Internet access required for HiveMQ cloud broker setup.

# 2. What Is MQTT? — Protocol Foundations

MQTT stands for Message Queuing Telemetry Transport. It is a lightweight, open-standard messaging protocol originally developed by IBM in 1999 for monitoring oil pipelines over satellite — environments where bandwidth was scarce and connections were unreliable. Today, MQTT is the dominant communication protocol for IoT, used by Amazon AWS IoT, Facebook Messenger, and millions of embedded devices worldwide.

## 2.1  Why MQTT for IoT?

Traditional HTTP is request-response: a client asks, a server answers, the connection ends. For IoT devices continuously reporting sensor readings or receiving commands, HTTP is inefficient. MQTT was designed specifically to overcome these limitations with a persistent connection and push-based messaging.

| Feature | HTTP (Web) | MQTT (IoT) |
| --- | --- | --- |
| Protocol model | Request-Response (Pull) | Publish-Subscribe (Push) |
| Connection | New connection per request | Persistent TCP connection |
| Bandwidth | High (~500 bytes minimum) | Very low (2-byte minimum packet) |
| Direction | Client initiates only | Any party can send at any time |
| Multiple receivers | One response per request | One message to many subscribers |
| Reliability | None built-in | QoS levels 0, 1, 2 available |
| Best for | Web pages, REST APIs | IoT sensors, home automation |

💡 Real-World MQTT Analogy - Think of MQTT like a radio broadcast. Publishers are stations broadcasting on a frequency (topic). - Subscribers are radios tuned to that frequency — they receive the broadcast automatically. - The broker is the radio tower — it receives transmissions and retransmits to all listeners. - The publisher does not know or care who is listening. New listeners tune in without any changes to the station.

## 2.2  MQTT vs Other IoT Protocols

| Protocol | Transport | Direction | Best For | Overhead |
| --- | --- | --- | --- | --- |
| MQTT | TCP | Bidirectional pub/sub | IoT telemetry & control | Very low (2 bytes min) |
| HTTP/REST | TCP | Request-response | Web APIs, configuration | Medium (~500 bytes) |
| WebSocket | TCP | Full-duplex persistent | Real-time dashboards | Low (after handshake) |
| CoAP | UDP | Request-response | Ultra-constrained devices | Very low |
| AMQP | TCP | Message queue | Enterprise messaging | Medium |

# 3. MQTT Core Concepts

MQTT is built on four fundamental concepts:

- Publish/Subscribe,

- Messages,

- Topics,

- and the Broker.

Mastering these is the foundation of all MQTT development.

## 3.1  Publish / Subscribe

Any MQTT client can publish messages to topics or subscribe to topics to receive messages. Publishers and subscribers never communicate directly — all messages pass through the broker. A Pico W publishing temperature data can simultaneously serve a dashboard, Home Assistant, a data logger, and a mobile app without any changes to its code.

- For example, Device 1 publishes on a topic;

- Device 2 is subscribed to the same topic that Device 1 is publishing in;

- So, Device 2 receives the message.

## 3.2  Messages

An MQTT message carries a payload (the data) and metadata. The payload is raw bytes and can contain any format: plain text, numbers, JSON, or binary data.

| Payload | The actual data. Any byte string: b'25.3', b'{"temp":25.3}', b'on' |
| --- | --- |
| Topic | Routing address — determines which subscribers receive the message. |
| QoS | Delivery guarantee level: 0 (at most once), 1 (at least once), 2 (exactly once). |
| Retain | If True, broker stores last message. New subscribers receive it immediately. |
| DUP flag | Marks a retransmitted message (used internally under QoS 1 and 2). |

## 3.3  Topics

Another important concept is the topics. Topics are the way you register interest in incoming messages or how you specify where you want to publish the message.

Topics are hierarchical strings using / as a separator. They represent the logical structure of your IoT system.

Note: topics are case-sensitive, which makes the following topics different.

If you would like to turn on a lamp in your home office using MQTT you can imagine the following scenario:

- A device publishes on and off messages on the home/office/lamp topic.

- You have a device that controls a lamp (it can be your Raspberry Pi Pico, or any other board or device). The Pico, that controls your lamp, is subscribed to that same topic: home/office/lamp.

- So, when a new message is published on that topic, the Pico receives the on or off messages and turns the lamp on or off.

The device that is publishing the messages can be another microcontroller board, or a Home Automation controller platform with MQTT support like Node-RED, Home Assistant, Adafruit IO, Domoticz, or OpenHAB, for example.

# Pico W topic hierarchy examples:

pico/temperature # Temperature reading

pico/humidity # Humidity reading

pico/pressure # Pressure reading

pico/led # LED control (subscribe)

pico/status # Online/offline status

# Home automation topic hierarchy:

home/office/temperature

home/office/lamp # Control lamp in office

home/bedroom/temperature

# Wildcard subscriptions:

pico/# # All pico/ topics (multi-level wildcard)

home/+/temperature # Temperature in ANY room (single-level wildcard)

⚠️ Topics Are Case Sensitive - pico/temperature and pico/Temperature and Pico/Temperature are three different topics. - Always use consistent lowercase naming in your topic hierarchy. - A subscriber to pico/temperature will NOT receive messages published to pico/Temperature.

## 3.4  The Broker

Finally, another important concept is the broker.

The MQTT broker is the central hub that receives all published messages, filters by topic, and forwards each message to all subscribed clients. It also handles authentication, session state, retained messages, and Last Will.

There are several brokers you can use. For example:

- Cloud MQTT Broker solutions: you can use commercial MQTT broker solutions like HiveMQ, for example. You don’t need to set up anything. You just need to create an account and you’re ready to go (this is the one we’ll use in this tutorial).

- Local MQTT broker: you can install an MQTT broker locally on your computer or on your Raspberry Pi. The Mosquitto MQTT broker hosted on a Raspberry Pi is widely used in many hobbyist projects and it’s also the solution we use more often.

- Cloud MQTT Broker: as an alternative to the previous solution, you can also install MQTT broker on your own cloud server.

In summary:

- MQTT is a communication protocol very useful in Internet of Things projects;

- In MQTT, devices can publish messages on specific topics and can be subscribed to topics to receive messages;

- You need a broker when using MQTT. It receives all the messages and sends them to the subscribed devices.

| Core function | Receive, filter, and forward messages between all clients. |
| --- | --- |
| Authentication | Username/password verification. TLS/SSL for encrypted connections. |
| Retained messages | Stores the last message per topic for new subscribers. |
| Last Will (LWT) | Sends a predefined message when a client disconnects unexpectedly. |
| Session state | Remembers persistent subscriptions across client reconnections. |

## 3.5  Quality of Service (QoS)

| QoS | Name | Guarantee | Use Case | Cost |
| --- | --- | --- | --- | --- |
| 0 | At Most Once | No guarantee. May be lost. | High-frequency sensors where occasional loss is OK. | Zero overhead |
| 1 | At Least Once | Delivered, possibly multiple times. | Commands where delivery matters but duplicates are tolerable. | ACK packet |
| 2 | Exactly Once | Delivered exactly once. No duplicates. | Financial, critical actuators (door lock, alarm). | 4-packet handshake |

# 4. Setting Up the MQTT Broker

To use MQTT, you need an MQTT broker. The broker receives all MQTT messages and sends them to all subscribed clients.

MQTT Broker Solutions

There are many MQTT broker solutions you can use. Here we’ll describe the ones we’re more familiar with:

- Mosquitto MQTT broker installed on a Raspberry Pi: this is an open-source MQTT broker that you can install locally on your Raspberry Pi (the Raspberry Pi computer, not the Pico board). We’ve been using this option a lot and it always worked well for us.

- Mosquitto MQTT broker installed on a cloud server: a great alternative if you want your broker to be accessible worldwide.

- HiveMQ Broker: this is a cloud MQTT broker service. You just need to create an account and choose a plan. They provide a free plan that is suitable for most IoT hobbyist projects.

| Broker | Type | Setup Effort | Internet Required | Best For |
| --- | --- | --- | --- | --- |
| HiveMQ Cloud | Cloud (Managed) | None — just register | Yes | Quick start, learning, cloud IoT |
| Mosquitto on RPi | Local (Self-hosted) | Medium | No | Offline use, home automation lab |
| Mosquitto on VPS | Cloud (Self-hosted) | High | Yes (always-on) | Production, remote worldwide access |
| Adafruit IO | Cloud (Managed) | Low | Yes | Beginners, built-in dashboard |

For simplicity, in this guide we’ll use HiveMQ because you don’t need to install or configure anything like in the Mosquitto MQTT broker. You just need to create an account, set up a cluster and you’re ready to go. Alternatively, if you want to use Mosquitto broker or any other broker of your choice, don’t worry. The code will be compatible with any broker as long as you fill in the right broker details.

Regardless of the MQTT broker you use, you need to have the MQTT URL, username, and password before proceeding to the following sections.

## 4.1  Option A — HiveMQ Cloud (Recommended)

In this section, we’ll show you how to set up your HiveMQ MQTT broker.

HiveMQ provides a free cloud MQTT broker with no software installation required. Simply create an account and you are ready.

### HiveMQ Setup — Step by Step

- Go to hivemq.com and click Start free.

- Select the HiveMQ Cloud free plan.

- Create an account or log in. Complete your profile.

- A default cluster is created automatically. Click Manage Cluster.

- Copy and save the Cluster URL (e.g., abc123.s1.eu.hivemq.cloud). This is your mqtt_server.

- Click the Access Management tab.

- Enter a username and password. Set permission to Publish and Subscribe.

- Finally, Click Create Credential.

✅ Record These Details After Setup mqtt_server = b'YOUR_CLUSTER.hivemq.cloud' mqtt_port = 8883 (TLS/SSL port — HiveMQ requires SSL) mqtt_username = b'your_username' mqtt_password = b'your_password'

If your credentials are successfully created, your MQTT broker is set up.

Make sure you have the following information before proceeding to the next section:

- MQTT Server:

  - Cluster URL (for HiveMQ)

  - MQTT broker IP address or URL (if you’re using a different MQTT Broker)

- MQTT username

- MQTT password

## 4.2  Option B — Mosquitto on Raspberry Pi (Local)

# Install Mosquitto on Raspberry Pi (Debian/Ubuntu):

sudo apt update && sudo apt install -y mosquitto mosquitto-clients

sudo systemctl enable mosquitto && sudo systemctl start mosquitto

# Edit config to allow remote connections:

sudo nano /etc/mosquitto/mosquitto.conf

# Add: listener 1883

# Add: allow_anonymous true (for testing only!)

# Restart and get Raspberry Pi IP:

sudo systemctl restart mosquitto

hostname -I # Use this IP as mqtt_server in config.py

# Test from another terminal:

mosquitto_sub -h localhost -t 'pico/#'

mosquitto_pub -h localhost -t 'pico/test' -m 'hello'

⚠️ Security Note allow_anonymous true is acceptable on a private local network for learning. NEVER use allow_anonymous true on a public-facing broker. For production: always use username/password authentication AND TLS encryption.

# 5. Installing MQTT MicroPython Modules

MicroPython does not include an MQTT client in its standard library. We install two modules: umqtt.simple (core MQTT client) and umqtt.robust (extends simple with automatic reconnection). Both are lightweight and designed for microcontrollers.

| Module | File | Purpose | When to Use |
| --- | --- | --- | --- |
| umqtt.simple | umqtt/simple.py | Core MQTT client: connect, publish, subscribe, check. | Basic projects, stable connections. |
| umqtt.robust | umqtt/robust.py | Extends simple with auto-reconnect on network failure. | Production, unstable networks. |

## 5.1  Installation via Thonny IDE

### Step 1 — Create the umqtt Directory

- Connect Raspberry Pi Pico W via USB and open Thonny IDE.

- Go to View > Files to show the filesystem panel.

A new sidebar will show up with all the files on the Raspberry Pi Pico filesystem.

- In the Pico W filesystem panel, right-click and select New directory...

- This new directory should be called umqtt. Click Ok.

The new directory will show up on the left sidebar.

### Step 2 — Create simple.py

- Right-click the umqtt folder and select New file... Name it simple.py.

- Download simple.py from: github.com/RuiSantosdotme/Random-Nerd-Tutorials/blob/master/Projects/Raspberry-Pi-Pico/MicroPython/mqtt/umqtt/simple.py

- Copy the entire code into the file and press Ctrl+S to save.

Now, if you expand the umqtt folder, you’ll see that the simple.py file is there.

### Step 3 — Create robust.py

- Right-click the umqtt folder and select New file... Name it robust.py.

- Download robust.py from the same GitHub repository directory.

- Copy the code into the file and save.

- After copying the code to the robust.py file, save the code. At this moment, you should have the umqtt folder with the simple.py and robust.py files inside.

The modules required for MQTT were successfully uploaded to the Raspberry Pi Pico.

# Expected Pico W filesystem after installation:

/ (root)

├── umqtt/

│ ├── simple.py # Core MQTTClient class

│ └── robust.py # Auto-reconnect wrapper

└── config.py # Created in next section

## 5.2  Alternative: mpremote mip install

# For MicroPython v1.20+, install from terminal:

pip install mpremote

mpremote mip install umqtt.simple

mpremote mip install umqtt.robust

## 5.3  Key umqtt.simple Methods

| MQTTClient(id, server, port, user, password, ssl) | Constructor. Creates MQTT client with all connection parameters. |
| --- | --- |
| .connect(clean_session=True) | Connect to broker. clean_session=True = no persistent session. |
| .publish(topic, msg, retain, qos) | Publish msg to topic. Both must be bytes (b'...' prefix). |
| .set_callback(func) | Register function called on each incoming message. MUST be set before subscribe(). |
| .subscribe(topic, qos) | Subscribe to a topic. set_callback must be called first. |
| .check_msg() | Non-blocking: check for messages, return immediately if none. |
| .wait_msg() | Blocking: halt until a message arrives. |
| .set_last_will(topic, msg, retain, qos) | Register LWT message. MUST be called before connect(). |
| .ping() | Send PINGREQ to keep connection alive. |
| .disconnect() | Gracefully close the MQTT connection. |

# 6. Project Setup — config.py & Wi-Fi Helper

Before writing application code, we establish a clean project structure.

A config.py file stores all credentials separately, making it easy to update without touching application logic.

We’ll create a configuration file to save the SSID, password, and your MQTT broker details: URL, username, and password.

## 6.1  config.py — Credentials File

Create a new file called config.py on Thonny IDE and copy the following code:

# config.py — Save to ROOT of Pico W filesystem (NOT inside umqtt/)

# Wi-Fi Settings (regular strings, no b prefix)

wifi_ssid = 'REPLACE_WITH_YOUR_SSID'

wifi_password = 'REPLACE_WITH_YOUR_PASSWORD'

# MQTT Settings — MUST be bytes (b prefix required!)

# For HiveMQ Cloud (SSL, port 8883):

mqtt_server = b'YOUR_CLUSTER.hivemq.cloud'

mqtt_port = 8883

mqtt_username = b'YOUR_USERNAME'

mqtt_password = b'YOUR_PASSWORD'

# For local Mosquitto (no SSL, port 1883):

# mqtt_server = b'192.168.1.79'

# mqtt_port = 1883

# mqtt_username = b''

# mqtt_password = b''

⚠️ Bytes vs Strings — Critical Difference MQTT server, username, and password MUST be bytes objects (b'...' prefix). The umqtt library passes them directly to the TCP socket as binary data. Wi-Fi SSID and password are normal strings (no b prefix) — they go to the network module. Common mistake: TypeError if you forget the b prefix on MQTT credentials.

Then, go to File > Save as… and select Raspberry Pi Pico. Save the file as config.py (overwrite any existing files with the same name). This file should be saved on the root of the Raspberry Pi Pico and not inside the umqtt folder.

## 6.2  Wi-Fi Connection Helper

# Wi-Fi helper — include in each project script

import network

from time import sleep

import config

# Init Wi-Fi Interface

def initialize_wifi(ssid, password):

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

# Connect to the network

wlan.connect(ssid, password)

# Wait for Wi-Fi connection

connection_timeout = 10

while connection_timeout > 0:

if wlan.status() >= 3:

break

connection_timeout -= 1

print('Waiting for Wi-Fi connection...')

sleep(1)

# Check if connection is successful

if wlan.status() != 3:

return False

else:

print('Connection successful!')

network_info = wlan.ifconfig()

print('IP address:', network_info[0])

return True

## 6.3  Generating a Unique Client ID

Every MQTT client must have a unique Client ID. Duplicate IDs cause the broker to disconnect the earlier client. Use the hardware MAC address to generate a guaranteed-unique ID.

from ubinascii import hexlify

import machine

# Generate unique client ID from Pico W hardware ID:

client_id = hexlify(machine.unique_id())

# Example: b'e66130ab1234' (hex of hardware unique ID)

# Or with a readable prefix:

mac = hexlify(machine.unique_id()).decode()

client_id = ('picow_' + mac[:6]).encode()

# Example: b'picow_e66130'

# 7. Project A1 — Publishing MQTT Messages (BME280 Sensor)

The Pico W reads temperature, humidity, and pressure from a BME280 sensor and publishes each reading to a dedicated MQTT topic every 10 seconds. Any subscribed client receives these readings in real time.

## 7.1  Hardware Wiring — BME280

| BME280 SDA | GPIO 4 (Pin 6) on Pico W |
| --- | --- |
| BME280 SCL | GPIO 5 (Pin 7) on Pico W |
| BME280 VCC | 3.3V (Pin 36) on Pico W |
| BME280 GND | GND (Pin 38) on Pico W |

💡 No BME280? Use Random Values for Testing import random temperature = str(round(random.uniform(20.0, 35.0), 2)) + 'C' humidity = str(round(random.uniform(40.0, 80.0), 2)) + '%' pressure = str(round(random.uniform(990.0, 1020.0), 2)) + 'hPa' This lets you test all MQTT publish logic without specific hardware.

## 7.2  Complete Publisher Script

The following code gets sensor data from the BME280 sensor and publishes the readings on different MQTT topics.

# Raspberry Pi Pico W — MQTT Publisher (BME280 Sensor Data)

# Source: Random Nerd Tutorials

# https://randomnerdtutorials.com/raspberry-pi-pico-w-mqtt-micropython/

from machine import Pin, I2C

from time import sleep

import network

from umqtt.simple import MQTTClient

from ubinascii import hexlify

import machine, config, BME280

# MQTT Topic constants — bytes required

TOPIC_TEMP = b'pico/temperature'

TOPIC_HUMIDITY = b'pico/humidity'

TOPIC_PRESSURE = b'pico/pressure'

def connect_wifi():

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

if not wlan.isconnected():

wlan.connect(config.wifi_ssid, config.wifi_password)

timeout = 10

while not wlan.isconnected() and timeout > 0:

sleep(1); timeout -= 1

if wlan.isconnected():

print('Wi-Fi connected. IP:', wlan.ifconfig()[0]); return True

return False

def connect_mqtt():

client_id = hexlify(machine.unique_id())

client = MQTTClient(

client_id = client_id,

server = config.mqtt_server,

port = config.mqtt_port,

user = config.mqtt_username,

password = config.mqtt_password,

keepalive = 60,

ssl = True,

ssl_params = {'server_hostname': config.mqtt_server}

)

client.connect()

print('Connected to broker:', config.mqtt_server)

return client

# Initialise I2C and BME280

i2c = I2C(0, sda=Pin(4), scl=Pin(5), freq=400000)

bme = BME280.BME280(i2c=i2c)

if not connect_wifi():

raise RuntimeError('Wi-Fi connection failed')

client = connect_mqtt()

while True:

try:

# Read sensor (BME280 library returns strings like '25.23C')

temperature = bme.temperature

humidity = bme.humidity

pressure = bme.pressure

print(f'Temp:{temperature} Hum:{humidity} Press:{pressure}')

# Publish — payload must be bytes (.encode() converts string to bytes)

client.publish(TOPIC_TEMP, temperature.encode(), retain=True)

client.publish(TOPIC_HUMIDITY, humidity.encode(), retain=True)

client.publish(TOPIC_PRESSURE, pressure.encode(), retain=True)

print('Published successfully')

except Exception as e:

print('Error:', e)

try: client = connect_mqtt()

except Exception as re: print('Reconnect failed:', re)

sleep(10)

We use the following MQTT topics to publish data:

- For the temperature — pico/temperature

- For the humidity — pico/humidity

- For the pressure — pico/pressure

And the workflow to publish data is the following:

- Connect the Pico to the internet;

- Connect to the MQTT broker;

- After connecting to the broker, we can continuously publish MQTT messages.

Let’s take a look at how the code works.

### Importing Libraries

First, we need to import the required libraries. We import the Pin and I2C from the machine module, and the BME280 to interface with the BME280 sensor. The network module to connect to Wi-Fi and the MQTTClient class from umqtt.simple to use MQTT functions.

from machine import Pin, I2C

from time import sleep

import network

from umqtt.simple import MQTTClient

import config

import BME280

### MQTT Topics

On the following variables we save the topics where we want to publish our messages (sensor readings).

# Constants for MQTT Topics

MQTT_TOPIC_TEMPERATURE = 'pico/temperature'

MQTT_TOPIC_PRESSURE = 'pico/pressure'

MQTT_TOPIC_HUMIDITY = 'pico/humidity'

### MQTT Details

In the following lines we set up the MQTT parameters to connect to our MQTT broker. We import the server URL, username, and password from the config.py file.

# MQTT Parameters

MQTT_SERVER = config.mqtt_server

MQTT_PORT = 0

MQTT_USER = config.mqtt_username

MQTT_PASSWORD = config.mqtt_password

MQTT_CLIENT_ID = b"raspberrypi_picow"

MQTT_KEEPALIVE = 7200

MQTT_SSL = True

MQTT_SSL_PARAMS = {'server_hostname': MQTT_SERVER}

The MQTT_CLIENT_ID should be a unique ID to identify the MQTT client. You can give it wherever name you want, but it needs to be unique among the MQTT clients connected to your broker. In our case, it’s raspberrypi_picow.

MQTT_CLIENT_ID = b"raspberrypi_picow"

If you’re using a local Mosquitto MQTT broker, you should set the MQTT_SSL parameter to False.

MQTT_SSL = False

### BME280 Sensor

In the following lines, we initialize the BME280 sensor and create a function called get_sensor_readings() that gets data from the BME280 and returns the temperature, humidity, and pressure.

# Initialize I2C communication

i2c = I2C(id=0, scl=Pin(5), sda=Pin(4), freq=10000)

# Initialize BME280 sensor

bme = BME280.BME280(i2c=i2c, addr=0x76)

def get_sensor_readings():

temp = bme.temperature[:-1]

hum = bme.humidity[:-1]

pres = bme.pressure[:-3]

return temp, hum, pres

### Initialize Wi-Fi

The initialize_wifi() function connects the Raspberry Pi Pico to a network. You need to initialize Wi-Fi to connect to the MQTT broker and exchange messages.

def initialize_wifi(ssid, password):

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

# Connect to the network

wlan.connect(ssid, password)

# Wait for Wi-Fi connection

connection_timeout = 10

while connection_timeout > 0:

if wlan.status() >= 3:

break

connection_timeout -= 1

print('Waiting for Wi-Fi connection...')

sleep(1)

# Check if connection is successful

if wlan.status() != 3:

return False

else:

print('Connection successful!')

network_info = wlan.ifconfig()

print('IP address:', network_info[0])

return True

### Connect to MQTT

The connect_mqtt() function connects to the MQTT broker using the details about the MQTT broker you’ve set up previously.

def connect_mqtt():

try:

client = MQTTClient(client_id=MQTT_CLIENT_ID,

server=MQTT_SERVER,

port=MQTT_PORT,

user=MQTT_USER,

password=MQTT_PASSWORD,

keepalive=MQTT_KEEPALIVE,

ssl=MQTT_SSL,

ssl_params=MQTT_SSL_PARAMS)

client.connect()

return client

except Exception as e:

print('Error connecting to MQTT:', e)

raise  # Re-raise the exception to see the full traceback

### Publish MQTT Messages

The publish_mqtt() function publishes a message on a topic. Pass as argument the topic and the message you want to send.

def publish_mqtt(topic, value):

client.publish(topic, value)

print(topic)

print(value)

print("Publish Done")

### Publishing Sensor Readings

Now that we have all functions and variables defined, we can finally connect to the internet and to the broker to start publishing messages.

First, we try to connect to Wi-Fi using the SSID and password stored on the config.py file.

try:

if not initialize_wifi(config.wifi_ssid, config.wifi_password):

print('Error connecting to the network... exiting program')

If we succeed in connecting to Wi-Fi, we can connect to the MQTT broker.

else:

# Connect to MQTT broker, start MQTT client

client = connect_mqtt()

After connecting, we get new sensor data and save it on the temperature, humidity, and pressure variables.

# Read sensor data

temperature, humidity, pressure = get_sensor_readings()

Finally, we use the publish_mqtt() function to publish the readings on their specific topics. The message must be a string. So, we need to convert the sensor data using the str() function.

# Publish as MQTT payload

publish_mqtt(MQTT_TOPIC_TEMPERATURE, str(temperature))

publish_mqtt(MQTT_TOPIC_PRESSURE, str(pressure))

publish_mqtt(MQTT_TOPIC_HUMIDITY, str(humidity))

We publish new readings every 10 seconds.

sleep(10)

### Testing the Code

Run the previous code on your Raspberry Pi Pico. It will connect to the internet and start publishing messages every 10 seconds.

Now, we can check if we can receive the messages on an MQTT Client subscribed to those topics. HiveMQ provides a Web Client interface that allows you to subscribe and publish to topics for testing purposes.

Go to your HiveMQ cluster, and click on the Web Client tab. Insert your MQTT broker username and password and click Connect Client.

Subscribe to the topics that the Raspberry Pi Pico is publishing to. Fill in pico/temperature and then click on +Subscribe.

Repeat the process for pico/humidity and pico/pressure.

After subscribing to those topics, scroll down to the bottom of the page. You should start receiving the Raspberry Pi Pico MQTT messages.

And that’s it. You successfully published MQTT messages with the Raspberry Pi Pico. Now, you can use any other MQTT Client to subscribe to those messages and receive the data. For example, having Node-RED nodes or Adafruit IO widgets subscribed to MQTT topics and displaying the data on charts and gauges.

You can also have any other microcontroller subscribed to those topics to receive the data—it can be another Raspberry Pi Pico, an ESP32, an ESP8266, or another board.

## 7.3  Key Code Points

### Payload Must Be Bytes

# String to bytes — two approaches:

temperature = '25.23C'

client.publish(b'pico/temperature', temperature.encode()) # .encode() method

# For numeric values:

temp_float = 25.23

client.publish(b'pico/temperature', str(temp_float).encode())

# JSON payload (best practice for structured data):

import json

data = {'temperature': 25.23, 'humidity': 64.8, 'pressure': 1013.2}

client.publish(b'pico/sensors', json.dumps(data).encode(), retain=True)

### retain=True Explained

With retain=True, the broker stores this as the last known value. Any new subscriber connecting later will immediately receive it — even if the Pico W published 10 minutes ago and is currently sleeping. Essential for sensor data where subscribers may connect at any time.

### SSL for HiveMQ

HiveMQ Cloud requires SSL/TLS on port 8883. The ssl=True and ssl_params={'server_hostname': config.mqtt_server} parameters enable encrypted connection. For a local Mosquitto broker without SSL, simply omit both parameters (ssl defaults to False).

## 7.4  Testing with MQTT Tools

| Tool | Platform | How to Use |
| --- | --- | --- |
| MQTT Explorer | Windows/macOS/Linux | GUI browser for all topics. Shows live messages. Download: mqtt-explorer.com |
| HiveMQ Web Client | Any browser | Available inside your HiveMQ cluster dashboard under Web Client. |
| mosquitto_sub | Linux terminal | mosquitto_sub -h broker_ip -t 'pico/#' — shows all pico/ messages |
| Node-RED | Any | Drag MQTT input node, set broker/topic, connect to debug node. |

# 7. Project A2 — Publishing MQTT Messages (DHT22 Sensor)

To replace the BME280 with a DHT22 sensor, you only need to make a few modifications:

### Changes Required

| BME280 | DHT22 |
| --- | --- |
| Uses I2C (SDA/SCL) | Uses single GPIO pin |
| Measures Temperature, Humidity, Pressure | Measures Temperature and Humidity only |
| Requires BME280.py library | Requires dht.py library |
| Publishes Temperature, Humidity, Pressure | Publishes Temperature and Humidity |

### Hardware Connection

Raspberry Pi Pico W ↔ DHT22

| DHT22 | Pico W |
| --- | --- |
| VCC | 3.3V |
| DATA | GP15 |
| GND | GND |

Add a 10kΩ pull-up resistor between VCC and DATA.

### Revised MQTT Publisher Code (DHT22)

from machine import Pin

from time import sleep

import network

from umqtt.simple import MQTTClient

import config

import dht

# MQTT Topics

MQTT_TOPIC_TEMPERATURE = 'pico/temperature'

MQTT_TOPIC_HUMIDITY = 'pico/humidity'

# MQTT Parameters

MQTT_SERVER = config.mqtt_server

MQTT_PORT = 0

MQTT_USER = config.mqtt_username

MQTT_PASSWORD = config.mqtt_password

MQTT_CLIENT_ID = b"raspberrypi_picow"

MQTT_KEEPALIVE = 7200

MQTT_SSL = True

MQTT_SSL_PARAMS = {'server_hostname': MQTT_SERVER}

# Initialize DHT22

dht_sensor = dht.DHT22(Pin(15))

def get_sensor_readings():

try:

dht_sensor.measure()

temperature = dht_sensor.temperature()

humidity = dht_sensor.humidity()

return temperature, humidity

except Exception as e:

print("DHT22 Error:", e)

return None, None

def initialize_wifi(ssid, password):

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

wlan.connect(ssid, password)

timeout = 10

while timeout > 0:

if wlan.status() >= 3:

break

timeout -= 1

print("Waiting for WiFi...")

sleep(1)

if wlan.status() != 3:

return False

print("WiFi Connected")

print("IP Address:", wlan.ifconfig()[0])

return True

def connect_mqtt():

try:

client = MQTTClient(

client_id=MQTT_CLIENT_ID,

server=MQTT_SERVER,

port=MQTT_PORT,

user=MQTT_USER,

password=MQTT_PASSWORD,

keepalive=MQTT_KEEPALIVE,

ssl=MQTT_SSL,

ssl_params=MQTT_SSL_PARAMS

)

client.connect()

print("MQTT Connected")

return client

except Exception as e:

print("MQTT Connection Error:", e)

raise

def publish_mqtt(topic, value):

client.publish(topic, str(value))

print("Topic:", topic)

print("Value:", value)

print("Published Successfully")

try:

if not initialize_wifi(

config.wifi_ssid,

config.wifi_password

):

print("WiFi Connection Failed")

else:

client = connect_mqtt()

while True:

temperature, humidity = get_sensor_readings()

if temperature is not None:

publish_mqtt(

MQTT_TOPIC_TEMPERATURE,

temperature

)

publish_mqtt(

MQTT_TOPIC_HUMIDITY,

humidity

)

sleep(10)

except Exception as e:

print("Error:", e)

### MQTT Topics Published

| Topic | Example Payload |
| --- | --- |
| pico/temperature | 29.4 |
| pico/humidity | 67.2 |

Example:

Topic: pico/temperature
Payload: 29.4

Topic: pico/humidity
Payload: 67.2

If You Want to Keep the Pressure Topic

Since the DHT22 cannot measure pressure, you can either:

Option 1: Remove Pressure Completely

Delete:

MQTT_TOPIC_PRESSURE = 'pico/pressure'

and

publish_mqtt(MQTT_TOPIC_PRESSURE, str(pressure))

Option 2: Publish a Placeholder Value

MQTT_TOPIC_PRESSURE = 'pico/pressure'

publish_mqtt(MQTT_TOPIC_PRESSURE, "N/A")

# 8. Project B — Subscribing to MQTT Topics (LED Control)

The Pico W subscribes to pico/led and responds by turning the onboard LED on or off based on incoming messages. This is the receive side of MQTT — the Pico acts as a command consumer, accepting instructions from any MQTT client on the network.

## 8.1  Hardware

The onboard LED (Pin 'LED') requires no extra hardware. For an external LED: GPIO 15 via 220 Ohm resistor to GND.

Here’s the workflow to subscribe to MQTT topics.

- Connect the Raspberry Pi Pico to the internet;

- Connect to the MQTT broker;

- Subscribe to the MQTT topic;

- Create and assign a callback function that will run when a message is received;

- Create a loop that is constantly checking for new MQTT messages.

The following code subscribes to the pico/led topic and controls an LED according to the received message.

## 8.2  Complete Subscriber Script

from machine import Pin

from time import sleep

import network

from umqtt.simple import MQTTClient

import config

# Define LED

led = Pin('LED', Pin.OUT)

# Constants for MQTT Topics

MQTT_TOPIC_LED = 'pico/led'

# MQTT Parameters

MQTT_SERVER = config.mqtt_server

MQTT_PORT = 0

MQTT_USER = config.mqtt_username

MQTT_PASSWORD = config.mqtt_password

MQTT_CLIENT_ID = b'raspberrypi_picow'

MQTT_KEEPALIVE = 7200

MQTT_SSL = True   # set to False if using local Mosquitto MQTT broker

MQTT_SSL_PARAMS = {'server_hostname': MQTT_SERVER}

# Init Wi-Fi Interface

def initialize_wifi(ssid, password):

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

# Connect to the network

wlan.connect(ssid, password)

# Wait for Wi-Fi connection

connection_timeout = 10

while connection_timeout > 0:

if wlan.status() >= 3:

break

connection_timeout -= 1

print('Waiting for Wi-Fi connection...')

sleep(1)

# Check if connection is successful

if wlan.status() != 3:

return False

else:

print('Connection successful!')

network_info = wlan.ifconfig()

print('IP address:', network_info[0])

return True

# Connect to MQTT Broker

def connect_mqtt():

try:

client = MQTTClient(client_id=MQTT_CLIENT_ID,

server=MQTT_SERVER,

port=MQTT_PORT,

user=MQTT_USER,

password=MQTT_PASSWORD,

keepalive=MQTT_KEEPALIVE,

ssl=MQTT_SSL,

ssl_params=MQTT_SSL_PARAMS)

client.connect()

return client

except Exception as e:

print('Error connecting to MQTT:', e)

# Subcribe to MQTT topics

def subscribe(client, topic):

client.subscribe(topic)

print('Subscribe to topic:', topic)

# Callback function that runs when you receive a message on subscribed topic

def my_callback(topic, message):

# Perform desired actions based on the subscribed topic and response

print('Received message on topic:', topic)

print('Response:', message)

# Check the content of the received message

if message == b'ON':

print('Turning LED ON')

led.value(1)  # Turn LED ON

elif message == b'OFF':

print('Turning LED OFF')

led.value(0)  # Turn LED OFF

else:

print('Unknown command')

try:

# Initialize Wi-Fi

if not initialize_wifi(config.wifi_ssid, config.wifi_password):

print('Error connecting to the network... exiting program')

else:

# Connect to MQTT broker, start MQTT client

client = connect_mqtt()

client.set_callback(my_callback)

subscribe(client, MQTT_TOPIC_LED)

# Continuously checking for messages

while True:

sleep(5)

client.check_msg()

print('Loop running')

except Exception as e:

print('Error:', e)

Let’s take a quick look at the relevant sections of code for this example.

If you’re using a local Mosquitto MQTT broker, you should set the MQTT_SSL parameter to False.

MQTT_SSL = False

### Subscribe to MQTT Topics

We create a function called subscribe() that accepts as arguments the MQTT client and the topic we want to subscribe to.

def subscribe(client, topic):

client.subscribe(topic)

print('Subscribe to topic:', topic)

### Callback Function

We need to create a callback function that will run when a new message is received on a topic we are subscribed to. The topic and message arguments are automatically passed to the function when it is called.

def my_callback(topic, message):

# Perform desired actions based on the subscribed topic and response

print('Received message on topic:', topic)

print('Response:', message)

# Check the content of the received message

if message == b'ON':

print('Turning LED ON')

led.value(1)  # Turn LED ON

elif message == b'OFF':

print('Turning LED OFF')

led.value(0)  # Turn LED OFF

else:

print('Unknown command')

In this function, we check the content of the message. If it is ON, we’ll turn the Raspberry Pi Pico built-in LED ON.

if message == b'ON':

print('Turning LED ON')

led.value(1)  # Turn LED ON

If the message is OFF, it will turn the LED off.

elif message == b'OFF':

print('Turning LED OFF')

led.value(0)  # Turn LED OFF

If we receive any other messages, we print Unknown command.

else:

print('Unknown command')

### Connecting to Wi-Fi, MQTT Broker and Checking Messages

To subscribe to the MQTT broker and constantly check for incoming messages, first we need to connect to the internet.

The following lines try to connect to the internet:

try:

# Initialize Wi-Fi

if not initialize_wifi(config.wifi_ssid, config.wifi_password):

print('Error connecting to the network... exiting program')

If we succeed, we also connect to the MQTT broker.

client = connect_mqtt()

Then, assign the callback function by using the set_callback() and passing as argument the callback function we created previously my_callback.

client.set_callback(my_callback)

Finally, we can subscribe to MQTT topics. In this case, we’re just subscribing to the MQTT_TOPIC_LED, but you can subscribe to multiple topics.

subscribe(client, MQTT_TOPIC_LED)

Then, create a while loop to constantly check for incoming messages using the check_msg() method.

while True:

sleep(5)

client.check_msg()

print('Loop running')

You can also add other tasks to the loop. We check for new messages every five seconds, but you can use a smaller or longer delay time depending on how often you’re expecting to receive messages.

### Testing the Code

Run the previous code on your Raspberry Pi Pico. It will connect to the internet and subscribe to the MQTT topic.

Now, let’s publish some messages on the pico/led topic to control the LED using HiveMQ Web Client.

Go to your HiveMQ cluster Web Client. Insert your MQTT broker username and password and click Connect Client.

Scroll down the page to the Publish Message section.

Insert pico/led on the Topic Name field, and type ON in the Message field.

Finally, press the Publish button.

Your Raspberry Pi Pico will receive the message, and will turn the on-board LED on.

Go back to HiveMQ Client and send an OFF message.

The Raspberry Pi Pico will receive the message and turn off the LED.

And that’s it. You successfully subscribed to an MQTT topic and executed a different task according to the received message. Now, instead of HiveMQ Web Client, you can use any Home Automation or IoT platform with MQTT support that allows you to build a web interface with buttons or switches to send MQTT messages.

## 8.3  Critical Subscription Rules

### Rule 1 — Set Callback Before Subscribe

# WRONG — subscribe before callback:

client.subscribe(b'pico/led') # Error: callback not set

client.set_callback(callback)

# CORRECT — callback first, then subscribe:

client.set_callback(callback)

client.connect()

client.subscribe(b'pico/led') # Now callback is ready to handle messages

### Rule 2 — Compare Bytes, Not Strings

# WRONG — comparing bytes to string:

if msg == 'on': # Always False — msg is bytes, 'on' is str

# CORRECT — compare bytes to bytes:

if msg == b'on': # Correct

# OR decode first:

if msg.decode() == 'on': # Also correct

# OR decode and normalise case:

if msg.decode().lower() == 'on': # Handles 'ON', 'On', 'on'

### check_msg() vs wait_msg()

| Method | Behaviour | Use When |
| --- | --- | --- |
| check_msg() | Non-blocking. Returns immediately if no message waiting. | You need to do other work between checks (sensors, display, etc.) |
| wait_msg() | Blocking. Halts execution until a message arrives. | Device only needs to react to messages, nothing else running. |

## 8.4  Sending Test Commands

# Test from Linux terminal:

mosquitto_pub -h broker_ip -t 'pico/led' -m 'on'

mosquitto_pub -h broker_ip -t 'pico/led' -m 'off'

mosquitto_pub -h broker_ip -t 'pico/led' -m 'toggle'

# With authentication:

mosquitto_pub -h broker_ip -u username -P password -t 'pico/led' -m 'on'

# 9. Advanced MQTT Features

## 9.1  umqtt.robust — Automatic Reconnection

The umqtt.robust module is a drop-in replacement for umqtt.simple that adds automatic reconnection. If the network drops, robust catches the OSError, waits, and reconnects. Recommended for all deployed (unattended) devices.

# Just change the import — everything else is identical:

from umqtt.robust import MQTTClient # instead of umqtt.simple

# robust automatically retries publish() and wait_msg() on failure.

# check_msg() accepts an 'attempts' parameter:

client.check_msg(attempts=3) # Retry up to 3 times

## 9.2  Retained Messages

# Publish with retain — broker remembers this value:

client.publish(b'pico/temperature', b'25.3', retain=True)

# New subscriber connects 5 minutes later ->

# Immediately receives b'25.3' even though Pico W is sleeping.

# Clear retained message — publish empty payload:

client.publish(b'pico/temperature', b'', retain=True)

## 9.3  Last Will and Testament (LWT)

LWT lets the broker send a message on behalf of a client that disconnects abnormally (power loss, crash). Register the LWT before connecting.

# Set LWT BEFORE client.connect():

client.set_last_will(

topic = b'pico/status',

msg = b'offline',

retain = True, # Keep 'offline' until device reconnects

qos = 1

)

client.connect()

# Publish 'online' immediately after successful connect:

client.publish(b'pico/status', b'online', retain=True)

# Result: pico/status shows 'online' while running,

# automatically becomes 'offline' if device unexpectedly drops.

## 9.4  Multiple Topic Subscriptions & Wildcards

# Subscribe to multiple specific topics:

client.set_callback(multi_callback)

client.connect()

client.subscribe(b'pico/led')

client.subscribe(b'pico/buzzer')

# Or subscribe to all pico/ subtopics with wildcard:

client.subscribe(b'pico/#')

# Differentiate topics in callback:

def multi_callback(topic, msg):

if topic == b'pico/led':

handle_led(msg)

elif topic == b'pico/buzzer':

handle_buzzer(msg)

## 9.5  JSON Payloads

import json

# Publish structured data as JSON:

data = {'temperature': 25.3, 'humidity': 64.8, 'pressure': 1013.2}

client.publish(b'pico/sensors', json.dumps(data).encode(), retain=True)

# Subscriber — decode JSON:

def callback(topic, msg):

if topic == b'pico/sensors':

readings = json.loads(msg.decode())

temp = readings['temperature']

print(f'Temperature: {temp}C')

# 10. MQTT in Real IoT Systems

## 10.1  Integration Platforms

| Platform | Role | MQTT Integration | Free Tier |
| --- | --- | --- | --- |
| Node-RED | Visual flow programming / dashboard | Built-in MQTT in/out nodes | Yes (self-hosted) |
| Home Assistant | Home automation hub | Native MQTT integration | Yes (self-hosted) |
| Grafana | Data visualisation | Via InfluxDB or MQTT plugin | Yes (self-hosted) |
| Adafruit IO | Cloud IoT dashboard & storage | MQTT feed topics built-in | Yes (limited) |
| AWS IoT Core | Enterprise cloud IoT | MQTT as primary protocol | 12 months free tier |

## 10.2  Node-RED Flow — Receive Pico W Sensor Data

# Node-RED receives pico/temperature and displays on a gauge widget

# Flow: [MQTT in: pico/temperature] -> [Function node] -> [Gauge widget]

# Function node JavaScript to parse value:

# var raw = msg.payload; // e.g. '25.23C'

# msg.payload = parseFloat(raw); // → 25.23

# return msg;

# To send LED command from Node-RED button:

# [inject node: payload='on'] -> [MQTT out: pico/led]

## 10.3  Home Assistant YAML Configuration

# configuration.yaml — MQTT sensors and switch for Pico W

mqtt:

sensor:

- name: 'Pico Temperature'

state_topic: 'pico/temperature'

unit_of_measurement: 'C'

device_class: temperature

- name: 'Pico Humidity'

state_topic: 'pico/humidity'

unit_of_measurement: '%'

device_class: humidity

switch:

- name: 'Pico LED'

command_topic: 'pico/led'

payload_on: 'on'

payload_off: 'off'

state_topic: 'pico/status'

## 10.4  Complete System Architecture

# ╔══════════════════════════════════════════════════════════════╗

# ║ Complete IoT MQTT Architecture ║

# ╚══════════════════════════════════════════════════════════════╝

#

# [Pico W + BME280] [Pico W + LED] [Mobile App]

# PUBLISH: SUBSCRIBE: PUBLISH:

# pico/temperature pico/led pico/led

# pico/humidity

# pico/pressure

# │ ▲ │

# └──────────────────┐ │ ─────────────────┘

# ▼ │

# ┌────────────────────────────────────────┐

# │ MQTT BROKER │

# │ (HiveMQ Cloud / Mosquitto on RPi) │

# └────────────────────────────────────────┘

# │ │ │

# ▼ ▼ ▼

# [Node-RED] [Home Assistant] [MQTT Explorer]

# Dashboard Automations Debug / Testing

# 11. Featured Resource & Further Reading

⭐ FEATURED RESOURCE — Highly Recommended Random Nerd Tutorials: Raspberry Pi Pico W — Getting Started with MQTT (MicroPython) URL: https://randomnerdtutorials.com/raspberry-pi-pico-w-mqtt-micropython/ This is the primary source for this lecture and one of the most comprehensive beginner-friendly guides to MQTT on the Raspberry Pi Pico W with MicroPython. Written by Sara Santos and Rui Santos, it covers the complete workflow from first principles: introducing MQTT concepts clearly, setting up HiveMQ as a cloud broker with step-by-step screenshots, installing umqtt modules through Thonny IDE, creating the config.py credentials file, and providing two fully tested projects — one for publishing BME280 sensor readings and one for subscribing to control an LED. The tutorial includes real Thonny IDE output screenshots and a working demonstration, making it an invaluable follow-along reference for students new to MQTT and IoT communication. Why it stands out: Complete beginner-to-working-project coverage • Step-by-step screenshots • Two real hardware projects (publish + subscribe) • HiveMQ cloud setup guide • Actively maintained • Part of the full Pico W MicroPython eBook series

## 11.1  Continuation Tutorials

| Tutorial | URL | What It Adds |
| --- | --- | --- |
| MQTT with Node-RED | randomnerdtutorials.com/esp32-mqtt-publish-subscribe-arduino-ide/ | Visualise sensor data in a Node-RED dashboard |
| Mosquitto on Raspberry Pi | randomnerdtutorials.com/how-to-install-mosquitto-broker-on-raspberry-pi/ | Install and configure local broker |
| Cloud MQTT Broker | randomnerdtutorials.com/cloud-mqtt-mosquitto-broker-access-anywhere-digital-ocean/ | Deploy Mosquitto on a cloud VPS |
| BME280 Deep Dive | randomnerdtutorials.com/raspberry-pi-pico-bme280-micropython/ | BME280 library installation and I2C usage |
| Home Automation | randomnerdtutorials.com/home-automation-using-raspberry-pi/ | Control Pico W from Home Assistant automations |

## 11.2  Official References

- MQTT 5.0 Specification — docs.oasis-open.org/mqtt/mqtt/v5.0/

- MicroPython umqtt source — github.com/micropython/micropython-lib/tree/master/micropython/umqtt.simple

- HiveMQ Documentation — docs.hivemq.com

- Mosquitto Documentation — mosquitto.org/documentation/

- MQTT Explorer GUI tool — mqtt-explorer.com

- Node-RED MQTT nodes — nodered.org

- Home Assistant MQTT — home-assistant.io/integrations/mqtt/

# 12. Summary, Review Questions & Lab Exercises

## 12.1  Key Concepts Summary

| Concept | Key Point |
| --- | --- |
| MQTT | Lightweight pub/sub protocol. 2-byte minimum packet. Persistent TCP connection. |
| Publish | Send message to a topic. Broker routes to all subscribers. Payload = bytes. |
| Subscribe | Register interest in topic. callback() called on each arrival. |
| Topic | Hierarchical bytes string. Case-sensitive. / separator. + and # wildcards. |
| Broker | Central hub: HiveMQ (cloud) or Mosquitto (local). Routes and stores messages. |
| QoS 0 | At most once. No guarantee. Lowest overhead. Use for frequent sensor data. |
| QoS 1 | At least once. Guaranteed, possible duplicates. Use for control commands. |
| QoS 2 | Exactly once. Highest guarantee and overhead. Use for critical operations. |
| umqtt.simple | Core MQTT client module. connect(), publish(), subscribe(), check_msg(). |
| umqtt.robust | Extends simple with auto-reconnect. Use for deployed unattended devices. |
| config.py | Separate credentials file. MQTT values must be bytes (b prefix). |
| Retain | retain=True: broker stores last message, new subscribers receive it immediately. |
| Last Will | Pre-registered message sent by broker if client disconnects abnormally. |
| check_msg() | Non-blocking poll. Returns immediately. Use when doing other tasks in loop. |
| Bytes payload | .encode() converts string to bytes. .decode() converts bytes to string. |

## 12.2  Review Questions

### A — MQTT Concepts

- What does MQTT stand for? Describe its original use case and why it suits IoT.

- Explain publish/subscribe. How does it differ from HTTP request-response?

- A Pico W publishes to pico/temperature. Which device receives it: (a) all MQTT devices, (b) only devices subscribed to pico/temperature, (c) devices subscribed to pico/#? Explain.

- What are topics? Why are they case-sensitive? Design a topic hierarchy for a 3-room smart home with temperature, humidity, and lamp control.

- What is the broker's role? Compare HiveMQ Cloud and Mosquitto on Raspberry Pi.

- Describe QoS 0, 1, and 2. State one appropriate use case for each level.

### B — MicroPython Implementation

- What two umqtt modules are used and what is the key difference between them?

- Why must MQTT server, username, and password be bytes objects (b'...') in umqtt?

- Write code to create and connect an MQTTClient to a local Mosquitto broker at 192.168.1.79 port 1883 with no username/password.

- What must you do before calling client.subscribe()? Why will it fail if skipped?

- Explain check_msg() vs wait_msg(). Which would you use if you also need to read a sensor every second?

- Write the callback function that receives topic=b'pico/led' and msg=b'ON' and turns on the onboard LED.

### C — Advanced & Systems

- What does retain=True do on publish()? Describe a scenario where this is essential.

- Write code to configure an LWT that publishes b'offline' to b'pico/status' with retain=True when the Pico W disconnects unexpectedly.

- How would you publish temperature=25.3 and humidity=64.8 as a single JSON payload to b'pico/sensors'?

- A subscriber script stops receiving messages after several hours of operation. List two possible causes and their fixes.

## 12.3  Lab Exercises

### Lab 1 — Publisher with Random Values (Beginner, 30 min)

- Install umqtt.simple and umqtt.robust on your Pico W via Thonny IDE.

- Create config.py with your Wi-Fi and HiveMQ (or Mosquitto) credentials.

- Write a script that generates random temperature (20–35), humidity (40–80), and pressure (990–1020) and publishes to pico/temperature, pico/humidity, pico/pressure every 5 seconds.

- Open MQTT Explorer or HiveMQ Web Client and verify messages arrive every 5 seconds.

- Test retain=True: subscribe after stopping the publisher and verify you still receive the last values.

### Lab 2 — LED Subscriber (Beginner, 30 min)

- Connect an LED to GPIO 15 with a 220 Ohm resistor.

- Write the subscriber script from Section 8 and run it.

- From MQTT Explorer or mosquitto_pub, send 'on', 'off', and 'toggle' to pico/led.

- Verify the LED responds correctly. Modify the callback to handle both uppercase and lowercase variants.

### Lab 3 — Bidirectional: Publish + Subscribe (Intermediate, 60 min)

- Write a single script that both publishes temperature data every 10 seconds AND subscribes to pico/led for LED control.

- Add an LWT: register b'offline' on b'pico/status' before connect(), then publish b'online' after connect().

- Test simultaneously: send LED commands while the publish loop is running — both should operate independently.

### Lab 4 — Node-RED Dashboard (Advanced, 90 min)

- Install Node-RED on a computer (nodered.org/docs/getting-started/).

- Wire a BME280 and run the full publisher from Section 7.

- Create a Node-RED flow with 3 MQTT input nodes (one per sensor topic) feeding 3 Gauge dashboard widgets.

- Add a Node-RED button that publishes 'toggle' to pico/led from the browser dashboard.

## 12.4  Quick Reference

| Import | from umqtt.simple import MQTTClient OR from umqtt.robust import MQTTClient |
| --- | --- |
| Create | client = MQTTClient(client_id, server, port, user, password, keepalive, ssl) |
| Connect | client.connect() |
| Publish | client.publish(b'topic', b'payload', retain=False, qos=0) |
| Set callback | client.set_callback(my_func) — BEFORE subscribe() |
| Subscribe | client.subscribe(b'topic', qos=0) |
| Check msg | client.check_msg() (non-blocking) / client.wait_msg() (blocking) |
| Last Will | client.set_last_will(b'topic', b'msg', retain, qos) — BEFORE connect() |
| Ping | client.ping() |
| Disconnect | client.disconnect() |
| str to bytes | 'hello'.encode() → b'hello' |
| bytes to str | b'hello'.decode() → 'hello' |
| JSON publish | json.dumps(dict).encode() |
| JSON receive | json.loads(bytes.decode()) |
| Unique client ID | hexlify(machine.unique_id()) (from ubinascii import hexlify) |

## simple.py

# forked from: https://github.com/micropython/micropython-lib/tree/master/micropython/umqtt.simple

import usocket as socket

import ustruct as struct

from ubinascii import hexlify

class MQTTException(Exception):

pass

class MQTTClient:

def __init__(

self,

client_id,

server,

port=0,

user=None,

password=None,

keepalive=0,

ssl=False,

ssl_params={},

):

if port == 0:

port = 8883 if ssl else 1883

self.client_id = client_id

self.sock = None

self.server = server

self.port = port

self.ssl = ssl

self.ssl_params = ssl_params

self.pid = 0

self.cb = None

self.user = user

self.pswd = password

self.keepalive = keepalive

self.lw_topic = None

self.lw_msg = None

self.lw_qos = 0

self.lw_retain = False

def _send_str(self, s):

self.sock.write(struct.pack("!H", len(s)))

self.sock.write(s)

def _recv_len(self):

n = 0

sh = 0

while 1:

b = self.sock.read(1)[0]

n |= (b & 0x7F) << sh

if not b & 0x80:

return n

sh += 7

def set_callback(self, f):

self.cb = f

def set_last_will(self, topic, msg, retain=False, qos=0):

assert 0 <= qos <= 2

assert topic

self.lw_topic = topic

self.lw_msg = msg

self.lw_qos = qos

self.lw_retain = retain

def connect(self, clean_session=True):

self.sock = socket.socket()

addr = socket.getaddrinfo(self.server, self.port)[0][-1]

self.sock.connect(addr)

if self.ssl:

import ssl

self.sock = ssl.wrap_socket(self.sock, **self.ssl_params)

premsg = bytearray(b"\x10\0\0\0\0\0")

msg = bytearray(b"\x04MQTT\x04\x02\0\0")

sz = 10 + 2 + len(self.client_id)

msg[6] = clean_session << 1

if self.user is not None:

sz += 2 + len(self.user) + 2 + len(self.pswd)

msg[6] |= 0xC0

if self.keepalive:

assert self.keepalive < 65536

msg[7] |= self.keepalive >> 8

msg[8] |= self.keepalive & 0x00FF

if self.lw_topic:

sz += 2 + len(self.lw_topic) + 2 + len(self.lw_msg)

msg[6] |= 0x4 | (self.lw_qos & 0x1) << 3 | (self.lw_qos & 0x2) << 3

msg[6] |= self.lw_retain << 5

i = 1

while sz > 0x7F:

premsg[i] = (sz & 0x7F) | 0x80

sz >>= 7

i += 1

premsg[i] = sz

self.sock.write(premsg, i + 2)

self.sock.write(msg)

# print(hex(len(msg)), hexlify(msg, ":"))

self._send_str(self.client_id)

if self.lw_topic:

self._send_str(self.lw_topic)

self._send_str(self.lw_msg)

if self.user is not None:

self._send_str(self.user)

self._send_str(self.pswd)

resp = self.sock.read(4)

assert resp[0] == 0x20 and resp[1] == 0x02

if resp[3] != 0:

raise MQTTException(resp[3])

return resp[2] & 1

def disconnect(self):

self.sock.write(b"\xe0\0")

self.sock.close()

def ping(self):

self.sock.write(b"\xc0\0")

def publish(self, topic, msg, retain=False, qos=0):

pkt = bytearray(b"\x30\0\0\0")

pkt[0] |= qos << 1 | retain

sz = 2 + len(topic) + len(msg)

if qos > 0:

sz += 2

assert sz < 2097152

i = 1

while sz > 0x7F:

pkt[i] = (sz & 0x7F) | 0x80

sz >>= 7

i += 1

pkt[i] = sz

# print(hex(len(pkt)), hexlify(pkt, ":"))

self.sock.write(pkt, i + 1)

self._send_str(topic)

if qos > 0:

self.pid += 1

pid = self.pid

struct.pack_into("!H", pkt, 0, pid)

self.sock.write(pkt, 2)

self.sock.write(msg)

if qos == 1:

while 1:

op = self.wait_msg()

if op == 0x40:

sz = self.sock.read(1)

assert sz == b"\x02"

rcv_pid = self.sock.read(2)

rcv_pid = rcv_pid[0] << 8 | rcv_pid[1]

if pid == rcv_pid:

return

elif qos == 2:

assert 0

def subscribe(self, topic, qos=0):

assert self.cb is not None, "Subscribe callback is not set"

pkt = bytearray(b"\x82\0\0\0")

self.pid += 1

struct.pack_into("!BH", pkt, 1, 2 + 2 + len(topic) + 1, self.pid)

# print(hex(len(pkt)), hexlify(pkt, ":"))

self.sock.write(pkt)

self._send_str(topic)

self.sock.write(qos.to_bytes(1, "little"))

while 1:

op = self.wait_msg()

if op == 0x90:

resp = self.sock.read(4)

# print(resp)

assert resp[1] == pkt[2] and resp[2] == pkt[3]

if resp[3] == 0x80:

raise MQTTException(resp[3])

return

# Wait for a single incoming MQTT message and process it.

# Subscribed messages are delivered to a callback previously

# set by .set_callback() method. Other (internal) MQTT

# messages processed internally.

def wait_msg(self):

res = self.sock.read(1)

self.sock.setblocking(True)

if res is None:

return None

if res == b"":

raise OSError(-1)

if res == b"\xd0":  # PINGRESP

sz = self.sock.read(1)[0]

assert sz == 0

return None

op = res[0]

if op & 0xF0 != 0x30:

return op

sz = self._recv_len()

topic_len = self.sock.read(2)

topic_len = (topic_len[0] << 8) | topic_len[1]

topic = self.sock.read(topic_len)

sz -= topic_len + 2

if op & 6:

pid = self.sock.read(2)

pid = pid[0] << 8 | pid[1]

sz -= 2

msg = self.sock.read(sz)

self.cb(topic, msg)

if op & 6 == 2:

pkt = bytearray(b"\x40\x02\0\0")

struct.pack_into("!H", pkt, 2, pid)

self.sock.write(pkt)

elif op & 6 == 4:

assert 0

return op

# Checks whether a pending message from server is available.

# If not, returns immediately with None. Otherwise, does

# the same processing as wait_msg.

def check_msg(self):

self.sock.setblocking(False)

return self.wait_msg()

## robust.py

# forked from: https://github.com/micropython/micropython-lib/tree/master/micropython/umqtt.robust

import utime

from . import simple

class MQTTClient(simple.MQTTClient):

DELAY = 2

DEBUG = False

def delay(self, i):

utime.sleep(self.DELAY)

def log(self, in_reconnect, e):

if self.DEBUG:

if in_reconnect:

print("mqtt reconnect: %r" % e)

else:

print("mqtt: %r" % e)

def reconnect(self):

i = 0

while 1:

try:

return super().connect(False)

except OSError as e:

self.log(True, e)

i += 1

self.delay(i)

def publish(self, topic, msg, retain=False, qos=0):

while 1:

try:

return super().publish(topic, msg, retain, qos)

except OSError as e:

self.log(False, e)

self.reconnect()

def wait_msg(self):

while 1:

try:

return super().wait_msg()

except OSError as e:

self.log(False, e)

self.reconnect()

def check_msg(self, attempts=2):

while attempts:

self.sock.setblocking(False)

try:

return super().wait_msg()

except OSError as e:

self.log(False, e)

self.reconnect()

attempts -= 1