Classroom Lecture Programming the Raspberry Pi Pico with VS Code & MicroPython From first install to running code on real hardware

| Subject RPi Programming and Application | Level Beginner – Intermediate | Duration 90 – 120 min | Board Pico / Pico W |
| --- | --- | --- | --- |

|  | ℹ Based on / Improved from • Original source: Random Nerd Tutorials — 'Programming Raspberry Pi Pico with VS Code and MicroPython' • This lecture expands the original with instructor notes, concept explanations, deeper code exercises, troubleshooting tables, checkpoints, and extension tasks. |
| --- | --- |

In this guide, you’ll learn how to program the Raspberry Pi Pico board with MicroPython using VS Code Editor and the MicroPico extension (previously called Pico-W-Go).

# 1.  Lesson Overview & Learning Objectives

By the end of this lesson every student will be able to:

- Explain what MicroPython is and why it is well-suited for microcontrollers.

- Install Python, VS Code, and the MicroPico extension on their own machine.

- Flash MicroPython firmware onto a Raspberry Pi Pico using BOOTSEL mode.

- Create and configure a MicroPython project folder in VS Code.

- Write, run, and upload MicroPython programs to the Pico.

- Read and control digital I/O pins and use the onboard LED.

- Diagnose and fix the seven most common setup problems.

|  | 📋 Instructor Opening (5 min) - Ask: 'Who has programmed a microcontroller before? What language?' — gauge experience. - Show a Pico and a credit card side by side — point out the chip. Ask what it could be used for. - Demo: plug in a pre-programmed Pico that blinks an LED. Ask students to predict what the code looks like. |
| --- | --- |

# 2.  Background Concepts — What Is MicroPython?

## 2.1  MicroPython in Plain English

Python is one of the most popular programming languages in the world — it runs on laptops, servers, and data-centre clusters. MicroPython is a lean re-implementation of Python 3 designed to run on microcontrollers with as little as 16 KB of RAM.

The Raspberry Pi Pico has 264 KB of RAM and 2 MB of flash storage — far more than MicroPython's minimum — so it is an excellent host. The trade-off is speed: interpreted Python is slower than compiled C, but for most student projects the difference is imperceptible.

|  | 💡 Key Concept — Interpreted vs Compiled • Compiled languages (C, C++) translate source code to machine code before running — fast, but requires a build step. • Interpreted languages (Python, MicroPython) translate and execute line-by-line at runtime — slower but instant feedback via the REPL. • The REPL (Read-Eval-Print Loop) lets you type Python directly into the Pico and see results immediately — like a calculator for code. |
| --- | --- |

## 2.2  How MicroPython Compares

Use this table to explain to students why we are choosing MicroPython for this class:

| Feature | MicroPython | C / C++ | Arduino IDE |
| --- | --- | --- | --- |
| Language | Python 3 subset | C / C++ | C++ dialect |
| Ease of use | Beginner-friendly | Expert-level | Intermediate |
| Speed | Slower (interpreted) | Very fast | Fast (compiled) |
| REPL shell | Yes (interactive) | No | Serial Monitor only |
| Best for | Prototyping, learning | Production firmware | Arduino ecosystem |

## 2.3  The Raspberry Pi Pico — Hardware Overview

The Pico is built around the RP2040 chip, a dual-core ARM Cortex-M0+ processor running at up to 133 MHz, designed in-house by the Raspberry Pi Foundation. Key facts students should know:

- 2 × ARM Cortex-M0+ cores (we will only use one in MicroPython).

- 264 KB SRAM (RAM) — holds running programs and variables.

- 2 MB on-chip Flash — stores MicroPython firmware and your .py files.

- 26 multi-function GPIO pins — digital, PWM, SPI, I2C, UART, ADC.

- 3 × 12-bit ADC channels (GP26, GP27, GP28) and 1 internal temperature sensor.

- Micro-USB port — powers the board AND transfers data / code.

- Onboard LED connected to GPIO 25 (Pico) or controlled via the 'LED' alias (Pico W).

|  | 📋 Instructor Tip • Pass a real Pico around the class. Point out: the USB connector, the BOOTSEL button, the RP2040 chip, and the 40-pin header. • Draw a simple diagram on the whiteboard: USB → PC (power + data) → Pico flash. Then: Pico flash → runs main.py at boot. |
| --- | --- |

# 3.  Step 1 — Installing Prerequisites

## 3.1  Install Python 3.9 or Newer

MicroPico uses your host computer's Python installation to manage the serial connection. You need Python 3.9 or higher in your PATH.

- Open a browser and go to  python.org/downloads

- Download the latest stable release for your OS.

- Run the installer. CRITICAL: on Windows, tick 'Add Python to PATH' before clicking Install Now.

- After installation, verify it worked. Open a terminal and run:

python --version # Expected output: Python 3.12.x (or similar 3.9+)

- Install the required pyserial package:

pip install pyserial

|  | ⚠ Windows PATH Trap • If 'python' is not recognised after install, you forgot to tick 'Add Python to PATH'. • Fix: re-run the installer > Modify > tick 'Add Python to PATH' and continue. • Alternative fix: add the Python and Scripts folders manually to System > Environment Variables > Path. |
| --- | --- |

## 3.2  Install Visual Studio Code

VS Code is a free, open-source code editor by Microsoft. It runs on all major operating systems. Follow the column for your OS:

| Windows | macOS | Linux (Ubuntu) |
| --- | --- | --- |
| 1. Visit code.visualstudio.com 2. Click Download for Windows 3. Run the .exe installer 4. Tick "Add to PATH" and "Add Open with Code" 5. Complete the wizard and launch VS Code | 1. Visit code.visualstudio.com 2. Click Download for macOS 3. Open the downloaded .zip 4. Drag VS Code to /Applications 5. Launch from Applications folder | 1. Download the .deb file 2. Open a terminal and run: sudo apt install ./code_*.deb 3. Find VS Code in app menu |

|  | ℹ Why VS Code? - Autocomplete and IntelliSense — even for MicroPython built-ins, the extension provides type hints. - Integrated terminal — no need to switch between windows. - Extension marketplace — thousands of plugins including MicroPico, GitLens, themes, and linters. - Cross-platform — the same workflow on Windows, macOS, and Linux. |
| --- | --- |

|  | ✔ Checkpoint 1 — Prerequisites Installed • python --version prints 3.9 or higher in the terminal. • pip install pyserial completes without errors. • VS Code opens and shows the Welcome tab. |
| --- | --- |

# 4.  Step 2 — Flash MicroPython Firmware

Before writing any code, MicroPython must be installed on the Pico. This is a one-time process; the firmware stays on the board permanently until you overwrite it.

## 4.1  Download the Correct Firmware

Choose the right .uf2 file for your exact board variant:

| Board | Variant | Download URL |
| --- | --- | --- |
| Raspberry Pi Pico | Standard (no Wi-Fi) | micropython.org/download/rp2-pico |
| Raspberry Pi Pico W | Wi-Fi only | micropython.org/download/rp2-pico-w |
| Raspberry Pi Pico W | Wi-Fi + BLE | raspberrypi.com datasheets page |

## 4.2  Enter BOOTSEL (Bootloader) Mode

The Pico will only accept a new firmware file when it is placed into bootloader mode using the BOOTSEL button.

- Disconnect the Pico from USB completely.

- Locate the small white BOOTSEL button on the top of the Pico board.

- Hold the BOOTSEL button down firmly with one finger.

- While still holding BOOTSEL, plug the USB cable into your computer.

- Release BOOTSEL. A new drive called RPI-RP2 should appear in File Explorer / Finder.

|  | ⚠ If RPI-RP2 Does Not Appear • Most common cause: the USB cable only carries power, not data. Try a different cable. • Second cause: BOOTSEL was released too early. Try again — hold until the drive appears. • Third cause: USB hub interference. Connect directly to a port on the computer. • On Linux: the drive may mount at /media/$USER/RPI-RP2 — check with: ls /media/$USER/ |
| --- | --- |

## 4.3  Copy the Firmware

- Open the RPI-RP2 drive — it contains two files (INDEX.HTM and INFO_UF2.TXT). Ignore these.

- Drag and drop (or copy-paste) the .uf2 file you downloaded into the RPI-RP2 drive.

- The Pico will copy the file, reboot automatically, and the drive will disappear. This is normal.

- The board is now running MicroPython.

|  | 💡 What Just Happened? • The .uf2 format is a special bootloader-aware file format. When copied onto RPI-RP2, the Pico's bootloader writes each block to flash at the correct address. • After flashing, the RP2040 boots from flash address 0x10000000 and starts the MicroPython interpreter. • The MicroPython REPL is now running on the Pico, waiting for commands over USB serial. |
| --- | --- |

|  | ✔ Checkpoint 2 — Firmware Flashed • RPI-RP2 drive appeared after holding BOOTSEL. • The .uf2 file was copied successfully and the drive disappeared. • No error messages from your OS during the copy. |
| --- | --- |

# 5.  Step 3 — Install and Configure MicroPico Extension

## 5.1  Install the Extension

- Open VS Code.

- Click the Extensions icon in the left sidebar (or press Ctrl+Shift+X / Cmd+Shift+X).

- In the search box, type:  MicroPico

- Find the extension named MicroPico by paulober and click Install.

- Wait for installation to complete. If prompted, restart VS Code.

|  | ℹ About MicroPico - MicroPico was previously called Pico-W-Go. If you see references to the old name in older tutorials, they mean the same thing. - The extension connects to the Pico over USB serial, gives you a REPL terminal, lets you run files without uploading, and uploads files to the Pico filesystem. - It also provides IntelliSense stubs for MicroPython modules (machine, utime, etc.) so autocomplete works correctly. |
| --- | --- |

## 5.2  Create and Configure a Project Folder

MicroPico needs a workspace folder to store project configuration files. Follow these steps for every new project:

- Create a new folder on your PC, e.g.  RPi Pico with VS Code

- In VS Code: File > Open Folder and navigate to the folder you just created.

- With the Pico plugged in via USB, open the Command Palette: Ctrl+Shift+P (Cmd+Shift+P on Mac).

- Type  micropico  and select  MicroPico: Configure project or Initialize MicroPico project.

- A 'Project configuration complete' message should appear in the bottom-right corner.

- VS Code will create these files in your folder:

RPi Pico with VS Code/ .micropico ← marks folder as a MicroPico project .vscode/ settings.json ← MicroPython IntelliSense settings

## 5.3  The MicroPico Status Bar

After configuration, the bottom bar of VS Code shows MicroPico controls. Familiarise students with each button:

| Status Bar Item | What It Does |
| --- | --- |
| MicroPico (Connected) | Shows connection status. Click to toggle connection. |
| Run (▶) button | Executes the currently open file on the Pico without saving it permanently. |
| Stop (■) button | Interrupts any running program (sends Ctrl+C to the REPL). |
| Upload button | Copies the current file to the Pico's filesystem permanently. |
| All commands | Opens the full MicroPico command palette for file management and more. |

|  | ⚠ Connected vs Disconnected - The status bar must show 'MicroPico Connected' before you can run or upload files. - If it shows 'Disconnected': ensure the Pico is plugged in, MicroPython is flashed, and no other application (Thonny, PuTTY) has the serial port open. - • Click the status bar item to manually trigger a reconnect. |
| --- | --- |

|  | ✔ Checkpoint 3 — MicroPico Ready • MicroPico extension is installed with no errors in VS Code. • Project folder has been created and configured (.micropico file is present). • Status bar shows 'MicroPico Connected'. • The Run and Upload buttons are visible at the bottom of the window. |
| --- | --- |

# 6.  Step 4 — Your First Program: Blinking LED

## 6.1  Write the Code

Inside your project folder, create a new file called  blinking.py  and type the following code exactly:

# blinking.py # Blink the onboard LED every 0.5 seconds # Works on both Pico and Pico W from machine import Pin from time import sleep led = Pin('LED', Pin.OUT) # 'LED' works on both Pico and Pico W # Use Pin(25, Pin.OUT) if you only have original Pico print('Blinking LED — press Stop to end') while True: led.value(not led.value()) # toggle: 1 becomes 0, 0 becomes 1 sleep(0.5) # wait 500 milliseconds

## 6.2  Code Walkthrough

Go through each section with the class before running it:

| Code | Explanation |
| --- | --- |
| from machine import Pin | Imports the Pin class from MicroPython's machine module, which gives low-level access to GPIO pins. |
| from time import sleep | Imports the sleep function for creating delays (in seconds). |
| Pin('LED', Pin.OUT) | Creates a Pin object. 'LED' is a special alias that resolves to the correct GPIO on both Pico and Pico W. Pin.OUT sets direction to output. |
| print('...') | Sends text to the REPL terminal in VS Code so we can confirm the program started. |
| while True: | An infinite loop — the indented code inside runs forever until stopped. |
| led.value(not led.value()) | Reads the current LED state (0 or 1), inverts it (not 0 = 1, not 1 = 0), and writes the new value back — toggling the LED each iteration. |
| sleep(0.5) | Pauses for 0.5 seconds. Without this, the LED would toggle millions of times per second, appearing permanently on. |

## 6.3  Run the File (Test Mode)

- Ensure the Pico is connected and the status bar shows 'MicroPico Connected'.

- Click the Run button (▶) at the bottom of VS Code.

- Watch the onboard LED — it should blink on and off every half second.

- The REPL terminal should print:  Blinking LED — press Stop to end

- Press the Stop button (■) to halt the program.

|  | ℹ Run vs Upload — Important Distinction - Run: executes the file directly from your PC over USB. It is NOT saved to the Pico permanently. If you unplug the Pico, the program is gone. - Upload (main.py): copies the file to the Pico's flash storage. If the file is named main.py, MicroPython runs it automatically every time the Pico is powered on — even without a computer. - Rule of thumb: use Run for testing, use Upload (named main.py) for deployment. |
| --- | --- |

## 6.4  Upload for Permanent Deployment

- Create a new file in your project folder called  main.py.

- Copy the exact same blinking code into main.py.

- Open the Command Palette (Ctrl+Shift+P) and run: MicroPico: Upload current file to Pico.

                - or => In VS Code, right-click main.py in the Explorer panel.

- Select  Upload file to Pico.

- Confirm the upload in the MicroPico status bar.

- Wait for the success message.

- Unplug the Pico from USB. Connect it to a USB power bank or USB charger.

- The LED should start blinking immediately — no computer needed.

|  | ✔ Checkpoint 4 — First Program Working • blinking.py runs and the onboard LED blinks at 0.5 s intervals. • REPL prints the startup message. • main.py uploaded and LED blinks when Pico is powered from a power bank (no PC). |
| --- | --- |

# 7.  Step 5 — Going Further: GPIO Input & ADC

## 7.1  GPIO Quick Reference

Students should keep this table open while writing GPIO programs:

| MicroPython Call | What it Does | Example |
| --- | --- | --- |
| Pin(n, Pin.OUT) | Configure pin n as digital output | led = Pin(25, Pin.OUT) |
| Pin(n, Pin.IN, Pin.PULL_UP) | Input with internal pull-up resistor | btn = Pin(15, Pin.IN, Pin.PULL_UP) |
| pin.value(1) / pin.value(0) | Set output HIGH or LOW | led.value(1) |
| pin.toggle() | Flip output state | led.toggle() |
| pin.value() | Read digital input (0 or 1) | state = btn.value() |
| ADC(n) | Analog input on ADC-capable pin | sensor = ADC(26) |
| sensor.read_u16() | Read 16-bit ADC value (0-65535) | v = sensor.read_u16() |

## 7.2  Exercise A — Variable Blink Speed with a Button

Wire a push-button between GP15 and GND (the internal pull-up resistor makes external resistors unnecessary). Then upload this program:

# button_blink.py # Normal speed: 0.5 s ¦ Button held: 0.1 s (fast blink) from machine import Pin from time import sleep led = Pin('LED', Pin.OUT) button = Pin(15, Pin.IN, Pin.PULL_UP) # PULL_UP: reads 1 when open, 0 when pressed while True: if button.value() == 0: # button pressed (pulled LOW) delay = 0.1 else: delay = 0.5 led.toggle() sleep(delay)

|  | 📋 Instructor Discussion Points • Ask: 'Why does the button read 1 when NOT pressed?' — leads to pull-up vs pull-down resistor explanation. • Ask: 'What would happen if we used Pin.PULL_DOWN instead?' — good thinking exercise. • Ask: 'If we want 3 blink speeds, how do we change the code?' — introduces elif. |
| --- | --- |

## 7.3  Exercise B — Read the Internal Temperature Sensor

The RP2040 has a built-in temperature sensor on ADC channel 4. This exercise reads it and prints the temperature every second:

# temperature.py # Read the RP2040 internal temperature sensor from machine import ADC import time sensor = ADC(4) # ADC channel 4 = internal temperature sensor VOLT_REF = 3.3 # Pico reference voltage CONVERSION = 65535 # 16-bit ADC max while True: raw = sensor.read_u16() # 0 – 65535 volts = raw * (VOLT_REF / CONVERSION) # convert to volts temp = 27 - (volts - 0.706) / 0.001721 # RP2040 formula print(f"Temperature: {temp:.1f} °C ¦ ADC raw: {raw}") time.sleep(1)

Expected output:

|  | ℹ The Formula Explained • The RP2040 datasheet gives: T = 27 - (V_ADC - 0.706) / 0.001721 • 0.706 V is the voltage at 27°C; 0.001721 V/°C is the sensor's slope. • The ADC returns a 16-bit value (0-65535) proportional to 0-3.3 V, so we scale it first. • The internal sensor is not calibrated for precision — expect ±2-3°C accuracy. |
| --- | --- |

# 8.  Troubleshooting — Common Problems & Fixes

Review this table with students before lab time. Most problems students encounter fall into one of these seven categories:

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| RPI-RP2 drive doesn't appear | BOOTSEL not held before plugging in, or charge-only cable | Hold BOOTSEL before connecting USB. Try a different cable labelled 'data'. Avoid cheap hub. |
| MicroPico shows 'Disconnected' | Pico not plugged in, wrong COM port, or MicroPython not flashed | Check Device Manager (Win) / ls /dev/tty.* (Mac). Re-flash firmware. Restart VS Code. |
| ModuleNotFoundError: machine | Running file with VS Code's built-in Python (not on the Pico) | Always use the MicroPico Run button at the bottom bar, not F5 or the green play button. |
| LED doesn't blink after reboot | File saved as blinking.py, not main.py | MicroPython only auto-runs main.py on boot. Rename and re-upload. |
| Syntax error in REPL on paste | Indentation mixed: spaces and tabs | Set VS Code to 'Spaces: 4'. Use Edit > Convert Indentation to Spaces before pasting. |
| pyserial missing error on connect | pyserial package not installed in host Python | Run: pip install pyserial in your OS terminal, then restart VS Code. |
| Upload hangs or times out | Pico filesystem full | Press Ctrl+C in REPL, then run: import os; os.listdir() to see what's on the board. Delete unneeded files. |

## 8.1  REPL Survival Commands

When something goes wrong, these REPL commands almost always help:

# Soft reset the Pico (keeps MicroPython running) # Press Ctrl+D in the REPL # Hard stop any running program # Press Ctrl+C in the REPL # List all files stored on the Pico import os print(os.listdir()) # Delete a file from the Pico os.remove('old_file.py') # Check free memory import gc gc.collect() print(gc.mem_free(), 'bytes free')

# 9.  Extension Challenges

For students who complete the main exercises ahead of schedule. No scaffolding is intentionally provided.

| # | Challenge | Description | Hint |
| --- | --- | --- | --- |
| E1 | SOS Morse Code | Make the LED flash SOS (···---···) in an infinite loop, then pause 2 s. | Each dot = 0.1 s on/off, dash = 0.3 s on/off. Use a list of durations. |
| E2 | Countdown Timer | Print a 10-to-0 countdown in the REPL with a 1-second pause between each number, then print 'LAUNCH!' | Use range(10, -1, -1) in a for loop. |
| E3 | LED Dimmer | Use PWM to fade the LED from off to full brightness smoothly and back, like breathing. | from machine import PWM. led.duty_u16(value) where value is 0–65535. |
| E4 | Dual-core Hello World | Run a blinking LED on core 0 AND print a counter on core 1 simultaneously. | import _thread. _thread.start_new_thread(func, ()) |

# 10.  Teacher Notes & Discussion Guide

## 10.1  Lesson Timing

| Time | Activity | Format |
| --- | --- | --- |
| 0 – 10 min | Section 1 & 2: Intro, concept discussion, show Pico hardware | Whole class |
| 10 – 25 min | Section 3: Install Python + VS Code. Students follow on own machines | Individual |
| 25 – 35 min | Section 4: Flash MicroPython firmware. Demonstrate first, then students | Pairs |
| 35 – 50 min | Section 5: Install MicroPico, configure project folder | Individual |
| 50 – 70 min | Section 6: Write, run and upload blinking LED. Walkthrough then students | Individual |
| 70 – 85 min | Section 7: GPIO exercises A and B | Individual / Pairs |
| 85 – 95 min | Section 9: Extension challenges for early finishers | Individual |
| 95 – 120 min | Debrief, show-and-tell, discussion questions (Section 10.2) | Whole class |

## 10.2  Discussion Questions

- MicroPython is interpreted. In what types of project would the speed difference compared to C matter? When would it not?

- The Pico's onboard LED is wired to GPIO 25 on the original Pico, but Pico W uses the wireless chip to drive it. How does using Pin('LED') help us write portable code?

- What is the difference between RAM and Flash on the Pico? What happens to each when you disconnect power?

- If you wanted the Pico to read a temperature and send it to a web server every minute, what else would you need? (Guides discussion toward Pico W Wi-Fi, HTTP/MQTT, etc.)

- The internal temperature sensor has ±2-3°C accuracy. In what real-world applications would this be acceptable, and in which would it not?

## 10.3  Differentiation

- Support: Provide a pre-wired breadboard example and printed step-by-step cards for sections 3-6. Pair struggling students with a peer mentor.

- Core: Follow the lecture as written, completing all exercises in sections 6 and 7.

- Extension: Extension challenges in section 9, especially E3 (PWM) and E4 (dual-core threading).

- Advanced: Challenge students to rewrite temperature.py to log readings to a CSV file on the Pico's filesystem using open() and write().

# 11.  Quick-Reference Card

Print or display at each workstation.

| VS Code Keyboard Shortcuts | REPL Quick Commands |
| --- | --- |
| Ctrl+Shift+P / Cmd+Shift+P → Command Palette Ctrl+Shift+X / Cmd+Shift+X → Extensions panel Ctrl+D (in REPL) → Soft reset Pico Ctrl+C (in REPL) → Stop running program Ctrl+` → Toggle integrated terminal | import os; os.listdir() → List files on Pico os.remove('file.py') → Delete a file import gc; gc.mem_free() → Check free RAM from machine import Pin → GPIO access from machine import ADC → Analog input from time import sleep → Delays |

— End of Lecture Notes —